Autograph Mini





Specifications

	Autograph Mini

	PERFORMANCE 

	Recommended amplifier power
	20 - 100 Watts

	Continuous power handling
	50 Watts RMS

	Frequency response
	68Hz - 54kHz -6dB

	Sensitivity
	88dB (2.83 Volts @ 1 metre)

	Nominal impedance
	8 Ohms

	DRIVE UNITS 

	Dual Concentric™ high frequency
	19mm (0.75”) titanium dome with Tulip WaveGuide™

	Dual Concentric™ low frequency
	100mm (4.00”) treated paper pulp cone with rubber surround. 33mm (1.33”) edge wound voice coil

	Dispersion
	90 degrees conical

	CROSSOVER 

	Frequency
	2.3kHz

	Type
	Passive low loss 2nd order compensated LF, 1st order compensated HF

	CONSTRUCTION 

	Enclosure type
	Rear ported

	Volume
	3.5L (213.5 cu.ins”)

	Dimensions
	345 x 210 x 130mm
(13.50 x 8.25 x 5.00”)

	Weight
	4.0kg (8.8 lbs)

	Finish
	Teak veneer with solid teak trim detail


